PRODUCT REQUIREMENTS DOCUMENT

Steel Beam Optimization for CIP (Cast-in-Place) Drilled-Hole Applications

A geometric-fit + AISC 360 / EN 1993-1-1 dual-code optimization framework, grid-search and Bayesian-refinement engines, and a React-based interactive calculator

| Field | Value |
| --- | --- |
| Document status | Draft framework — Phase 1 |
| Prepared for | Structural / Geotechnical Engineering — Foundations & Construction Technology |
| Related work | Helical pile optimization for light poles (LPILE p-y, ASTM D3966); Directionally Optimized H-Beam (DOHB) retaining wall proposal |
| Codes covered | AISC 360-16 (LRFD) and EN 1993-1-1 (Eurocode 3), and KS |
| Companion files | optimized_beam_aisc.py · optimized_steel_beam.py · bayesian_optimization.py · CIPBeamOptimizer.jsx · CIP_Beam_Optimizer.html |

Contents

Contents	1

1. Purpose & Background	1

1.1 Objectives	1

2. Scope	1

2.1 In scope — Phase 1	1

2.2 Out of scope — Phase 1 (see Section 10, Roadmap)	1

3. Input Parameters	1

4. Geometric Fit Methodology — Net Diagonal Length	1

4.1 Why the diagonal governs, not depth or flange width	1

4.2 Construction allowance values — verification note	1

4.3 Worked example	1

5. Comprehensive Process for Selecting the Optimum Cross Section	1

6. Selected Beam Capacity Calculation — AISC 360 and EN 1993-1-1	1

6.1 AISC 360-16 (LRFD)	1

6.2 EN 1993-1-1 (Eurocode 3)	1

6.3 Lateral-torsional buckling — the grout-bracing simplification	1

6.4 Code-to-code "Rosetta stone"	1

7. Optimization Program Architecture	1

7.1 Stage 1 — deterministic grid search (Phase 1, production)	1

7.2 Stage 2 — Bayesian refinement (Phase 2 / prototype, roadmap item)	1

7.3 React interactive calculator	1

7.4 File manifest	1

8. Assumptions & Limitations (Phase 1)	1

9. Acceptance Criteria — Phase 1	1

10. Roadmap	1

11. References	1

1. Purpose & Background

Cast-in-place (CIP) drilled-hole steel beam construction — used for soldier piles, augercast composite piles, and permanent retaining elements — installs a wide-flange steel beam into a pre-drilled hole before the annulus is filled with grout or structural concrete. The beam must satisfy two coupled constraints simultaneously: it must physically fit inside the drilled hole with adequate construction clearance, and it must carry the design lateral/moment demand once encased. Today this selection is typically done by hand, iterating between a geometry check and a code capacity check — slow, inconsistent across engineers, and rarely optimized for steel weight.

This PRD defines a repeatable, code-compliant, and optimization-driven framework for selecting the lightest admissible wide-flange section for a given hole diameter, construction allowance, and design demand, under both AISC 360 (US) and EN 1993-1-1 (Eurocode 3), and Korean Standard methods. It follows the same governing-load philosophy established on the helical pile foundation research (lateral/moment-dominant behavior, not axial) and is a natural structural companion to the Directionally Optimized H-Beam (DOHB) retaining-wall concept: DOHB addresses how the beam resists soil pressure once installed; this framework addresses whether — and how efficiently — that beam can physically get into the ground in the first place.

1.1 Objectives

- Formalize the geometric fit constraint ("net diagonal") that governs beam-in-hole installation, and make its derivation and assumptions explicit and auditable.

- Provide code (AISC / EN / KS) capacity-checked optimization so the same tool serves US and international projects.

- Automate section selection via exhaustive grid search over standard section libraries (guarantees a global optimum within the catalog).

- Establish a Phase 2 pathway (Bayesian optimization) for continuous / custom built-up sections and coupled hole-diameter–steel-weight cost optimization.

- Deliver a self-contained, team-shareable interactive calculator requiring no build tooling.

- Deliver a self-contained, team-shareable interactive section properties calculator requiring no build tooling.

2. Scope

2.1 In scope — Phase 1

- Doubly-symmetric wide-flange sections (AISC W-shapes; EN HE-B / HE-M / IPE series; KS), strong-axis bending.

- Geometric fit (net diagonal) check against drilled hole diameter and construction allowance.

- AISC 360-16: Chapter F2 flexure (compact sections, simplified Lr), Chapter G2.1 shear, Chapter H1 axial-flexure interaction (optional).

- EN 1993-1-1: §6.2.5 bending, §6.2.6 shear, §6.2.8 bending-shear interaction, §6.2.9 axial-flexure interaction (optional), §6.3.2 simplified LTB screening.

- Two lateral-bracing conditions: grout-braced in-service (Lb → 0) and unbraced construction/handling stage (Lb > 0, user-input).

- Exhaustive grid-search optimization (deterministic, catalog-bounded) and a Phase-2-track Bayesian refinement prototype.

2.2 Out of scope — Phase 1 (see Section 10, Roadmap)

- Biaxial bending, torsion, seismic detailing provisions, fatigue.

- Full AISC F2 LTB (Cw, J, rts, ho) and full EN 1993-1-1 Annex Mcr (It, Iw, C1) — Phase 1 uses conservative, documented simplifications (Section 6).

- Custom / built-up (welded) sections as a first-class catalog entry — targeted by the Phase 2 Bayesian layer, not the Phase 1 grid search.

- Corrosion-loss (sacrificial thickness) reduction of section properties — recommended as a Phase 2 input, not currently modeled.

- Soil-structure interaction / demand generation. This tool consumes Mu/Vu (or MEd/VEd) as inputs; generating those from p-y lateral analysis is the domain of the companion LPILE-based helical pile workflow and should be applied the same way here for wall/pole demand.

3. Input Parameters

All three tools (both grid-search optimizers and the React calculator) share the same input set so results are directly comparable across the AISC and EN paths.

| Parameter | Symbol | Units | Notes |
| --- | --- | --- | --- |
| Drilling hole diameter | D_hole | mm (in) | Set by auger/casing size or geotechnical requirement. |
| Construction allowance | C | mm (in) | Tiered default: 50 / 75 / 100 mm, or custom. See Section 4.2 — verify against project spec. |
| Allowance basis | — | toggle | Per-side (D_net = D_hole − 2C) or total clearance (D_net = D_hole − C). Per-side is the more common/conservative convention and is the tool default. |
| Design code | — | toggle | AISC 360-16 or EN 1993-1-1. |
| Steel grade | Fy / fy | ksi / MPa | AISC default Gr. 50 (Fy=50 ksi), alt. A36 (36 ksi). EN default S355 (355 MPa), alt. S235 (235 MPa). |
| Design moment | Mu / MEd | kip·in / kN·m | Factored, from governing lateral/moment analysis (see Section 2.2). |
| Design shear | Vu / VEd | kip / kN | Factored. |
| Design axial (optional) | Pu / NEd | kip / kN | Enables H1 / 6.2.9 interaction check; 0 = flexure-shear only. |
| Bracing condition | — | toggle | Grout-braced (Lb=0, governs in-service) or construction stage (Lb>0, user-input unbraced length). |

4. Geometric Fit Methodology — Net Diagonal Length

4.1 Why the diagonal governs, not depth or flange width

A wide-flange section can rotate about its longitudinal axis during lowering, plumbing, and final positioning in the hole — it is not guaranteed to stay orthogonally aligned. The largest cross-sectional dimension that can contact the hole wall at any rotation is therefore the corner-to-corner diagonal, not the section depth (d, h) or flange width (bf, b) individually:

diagonal = √(d² + bf²)          [AISC notation]
diagonal = √(h² + b²)          [EN notation]

The net diagonal length available inside the hole, after subtracting the construction allowance, is:

D_net = D_hole − 2·C        (allowance applied per side — tool default)
D_net = D_hole − C          (allowance applied as a single total clearance)

A section is geometrically admissible only if its diagonal is less than or equal to D_net. This single inequality is the first, non-negotiable filter applied before any strength check — a section that does not fit cannot be installed regardless of its capacity.

4.2 Construction allowance values — verification note

The three requested tiers (50 mm, 75 mm, 100 mm) were checked against public drilled-pile / soldier-pile specifications and industry practice literature. Finding: there is no single AISC, EN, or OSHA clause that mandates a beam-in-hole diagonal clearance — it is a construction-tolerance decision, typically set by the specifying agency or contractor based on:

- drilling/plumbness tolerance — commonly on the order of ±25 mm (1 in) per typical DOT drilled-pile special provisions;

- minimum grout/concrete cover over the steel for corrosion protection and bond development;

- clearance needed alongside the beam for a tremie pipe or grout tube during placement;

- larger holes and/or stricter plumbness tolerances generally warrant the larger allowance tier.

Recommendation: treat 50 / 75 / 100 mm as reasonable, commonly-seen default tiers for preliminary sizing, selectable in the tool — but confirm the governing value against the project's own drilled-pile specification or the contractor's means-and-methods submittal before finalizing a design. This is flagged directly in the calculator and in both Python optimizer docstrings so the assumption is never silently buried.

4.3 Worked example

| Item | Value |
| --- | --- |
| Hole diameter, D_hole | 610 mm (24.0 in) |
| Construction allowance, C (per side) | 75 mm (2.95 in) |
| Net diagonal available, D_net | 610 − 2(75) = 460 mm (18.11 in) |
| Admissible AISC sections (demand-dependent) | e.g., W8x31 (diagonal 287 mm / 11.31 in) — fits with margin |
| Admissible EN sections (demand-dependent) | e.g., IPE300 (diagonal 335 mm) — fits with margin |
| Admissible KS sections (demand-dependent) | e.g., H300 (diagonal 412 mm) — fits with margin |

5. Comprehensive Process for Selecting the Optimum Cross Section

The end-to-end procedure implemented by the optimizer scripts and the React calculator:

- Establish factored design demand (Mu/Vu or MEd/VEd, and Pu/NEd if applicable) from the governing lateral/moment analysis — consistent with the lateral-and-moment-dominant framing used on the helical pile light-pole work; do not size for axial alone.

- Confirm the drilled hole diameter from the geotechnical/drilling means-and-methods (auger size, casing OD, or minimum practical hole for the expected beam).

- Select the construction allowance tier (50 / 75 / 100 mm) or enter a project-specific value; confirm the per-side vs. total-clearance convention with the spec (Section 4.2).

- Compute the net available diagonal, D_net = D_hole − 2C (default) or D_hole − C.

- Run the geometric screen: for every section in the applicable library (AISC W-shapes or EN HE/IPE), compute diagonal = √(d²+bf²) and retain only sections with diagonal ≤ D_net.

- For each geometrically admissible section, determine the lateral-bracing condition: grout-braced in-service (Lb → 0, governs final design) vs. unbraced construction/handling stage (Lb > 0, governs only if it produces a lower capacity than the in-service case — check both and take the controlling case).

- Compute flexural capacity per the selected code (Section 6) and compare to Mu / MEd.

- Compute shear capacity per the selected code and compare to Vu / VEd; apply the bending-shear interaction reduction if VEd exceeds 0.5·Vpl,Rd (EN) or as required by AISC Chapter G.

- If axial load is present, apply the AISC H1 or EN 6.2.9 interaction check in place of the flexure-only check.

- Rank all sections that pass every check by unit weight (plf or kg/m) and select the lightest — this is the Phase 1 optimum. Optionally run the Bayesian refinement (Section 7.3) to test whether a custom/built-up section could do meaningfully better, when the standard catalog result is inefficient (utilization well below 1.0 across the board is a signal worth checking).

- QA/QC: verify the selected section's tabulated properties against the current AISC Steel Construction Manual or EN 10365 producer tables (Phase 1 libraries are indicative), or KS confirm availability/lead time with the fabricator, and document all assumptions (allowance tier, bracing condition, code edition, grade) on the calculation package.

6. Selected Beam Capacity Calculation — AISC 360 and EN 1993-1-1, KS

6.1 AISC 360-16 (LRFD)

Flexure, Chapter F2 (compact doubly-symmetric I-shapes, strong axis):

Mp = Fy · Zx
Lp = 1.76 · ry · √(E/Fy)
Lr ≈ π · ry · √(E/0.7Fy)                    [simplified — see 6.3]
Lb ≤ Lp:            Mn = Mp
Lp < Lb ≤ Lr:  Mn = Cb·[Mp − (Mp−0.7FySx)·(Lb−Lp)/(Lr−Lp)] ≤ Mp
Lb > Lr:            Mn = Cb·Mp·(Lr/Lb)² ≤ Mp     [elastic approx.]
φMn = 0.90 · Mn

Shear, Chapter G2.1 (unstiffened compact web, Cv1 = 1.0):

Aw = d · tw
φVn = 0.90 · 0.6 · Fy · Aw · Cv1

Combined axial + flexure, Chapter H1 (optional, if Pu > 0):

Pr/Pc ≥ 0.2:  Pr/Pc + (8/9)(Mr/Mc) ≤ 1.0     [H1-1a]
Pr/Pc < 0.2:  Pr/(2Pc) + (Mr/Mc) ≤ 1.0        [H1-1b]

6.2 EN 1993-1-1 (Eurocode 3)

Bending resistance, §6.2.5 (Class 1/2 sections):

Mc,Rd = Wpl,y · fy / γM0            (γM0 = 1.00)

Shear resistance, §6.2.6 (unstiffened web, simplified shear area):

Av ≈ A − 2·b·tf + tw·tf
Vpl,Rd = Av · (fy/√3) / γM0

Bending-shear interaction, §6.2.8 (applied when VEd > 0.5·Vpl,Rd):

ρ = (2·VEd/Vpl,Rd − 1)²
My,V,Rd = reduced Mc,Rd (fy in shear area factored by (1−ρ))

Axial-flexure interaction, §6.2.9.1 (optional, linear form for I-sections):

n = NEd / Npl,Rd ;  aw = min[(A − 2·b·tf)/A , 0.5]
n ≤ aw:  MN,y,Rd = Mc,Rd
n > aw:  MN,y,Rd = Mc,Rd · (1−n)/(1−0.5·aw)

6.3 Lateral-torsional buckling — the grout-bracing simplification

The single most consequential modeling decision in this framework: once the annulus around the beam is filled with grout or structural concrete and cured, the beam is continuously laterally and torsionally restrained by the surrounding fill. Both codes' LTB provisions (AISC F2.2, EN §6.3.2) exist to penalize capacity for UNBRACED length — with continuous bracing, Lb → 0 and Mn = Mp (AISC) / Mb,Rd = Mc,Rd (EN) govern directly, with no LTB reduction. This is the default "grout-braced" toggle state in both scripts and the calculator.

The only condition where LTB can actually govern is the temporary/construction stage — after the beam is set and plumbed but before grout placement and cure, when it may be laterally unsupported over some length (e.g., between guide frames, or the free length above grade). For that case only, both tools include a simplified LTB screening check:

- AISC: Lr approximated without the Cw/J/rts warping-torsion terms (conservative for preliminary screening).

- EN: Mcr approximated from Iz and an approximate warping constant Iw ≈ Iz·(h−tf)²/4, omitting the St. Venant torsion term (also conservative).

For any case where the construction-stage unbraced check governs and shows a low margin, or for final stamped design, replace these screening approximations with full AISC Manual Table 3-2/3-6 values or a full Eurocode Mcr calculation (hand calc, LTBeam, or FE).

6.4 Code-to-code "Rosetta stone"

| Concept | AISC 360 | EN 1993-1-1 |
| --- | --- | --- |
| Plastic section modulus | Zx | Wpl,y |
| Partial safety factor (member resistance) | φ (applied to Rn, e.g. φb=0.90) | γM0 / γM1 (divides Rk) |
| Plastic moment capacity | φMn = φ·Fy·Zx (short Lb) | Mc,Rd = Wpl,y·fy/γM0 |
| Shear capacity | φVn = φ·0.6·Fy·Aw | Vpl,Rd = Av·fy/√3/γM0 |
| Lateral-torsional buckling | Chapter F2 (Lp, Lr, Cb) | §6.3.2 (χLT, λLT, curve selection) |
| Axial-flexure interaction | Chapter H1 (H1-1a/b) | §6.2.9 / §6.3.3 |

7. Optimization Program Architecture

7.1 Stage 1 — deterministic grid search (Phase 1, production)

optimized_beam_aisc.py and optimized_steel_beam.py (Python — see file-naming note below) each hold a small, curated section library (16/15 common sizes) and exhaustively evaluate every section against the geometric fit test and the applicable code checks in Section 6. Because the catalog is small and finite, exhaustive search is the correct and fastest approach — it is guaranteed to find the true lightest admissible catalog section, with no risk of a local optimum. Typical run time is milliseconds.

Naming note: the brief specified "optimized_steel_beam.m" for the Eurocode grid search. The ".m" extension conventionally denotes MATLAB/Octave source, but the described content ("Python: grid search optimizer") is Python — the deliverable was written as optimized_steel_beam.py to match its actual language; the algorithm is directly transliterable to MATLAB if that runtime is specifically required.

7.2 Stage 2 — Bayesian refinement (Phase 2 / prototype, roadmap item)

bayesian_optimization.py demonstrates a dependency-free (numpy-only) Gaussian-process/UCB Bayesian optimization loop over a continuous (depth, flange width) proxy design space. It is intentionally NOT positioned to replace the grid search — for a 15-20 entry discrete catalog, grid search is already exhaustive and optimal. BO earns its place for:

- screening custom / built-up (welded) H-sections, where the design space is continuous rather than a fixed catalog;

- jointly optimizing beam selection against hole diameter / allowance within a feasible project range, to minimize total installed cost (drilling cost scales roughly with D_hole², steel cost scales with section weight) — a natural Phase 2 extension;

- cases where each capacity evaluation is expensive (e.g., calling out to LPILE/PLAXIS for soil-structure-interaction-based demand rather than a closed-form check), where minimizing the number of evaluations matters.

Current status: functional prototype, producing a continuous (d, bf) sizing target and a nearest-standard-catalog cross-check, but not yet wired into the Phase 1 acceptance criteria. Treat its output as illustrative / a Phase 2 seed, not a production selection path.

7.3 React interactive calculator

CIPBeamOptimizer.jsx (renders directly in Claude / any React environment) and CIP_Beam_Optimizer.html (a self-contained, build-tool-free standalone version using React + Babel via CDN, for direct team distribution/email/intranet hosting) implement the full Section 5 workflow client-side:

- Hole diameter, allowance (tiered chips + custom), and per-side/total toggle.

- AISC / EN /KS code toggle, driving unit system, section library, and formulas (Section 6, mirrored in JavaScript from the Python scripts for consistency).

- Live SVG diagram: drilled hole, net-diagonal boundary, and the current optimum section's silhouette drawn at its governing diagonal orientation — the single visual that answers "does it fit" at a glance.

- Ranked, color-coded results table (fit / no-fit, utilization ratios) with the lightest admissible section highlighted as the optimum.

7.3 React interactive beam property calculator

With the input values of H, B, tw, tf, R(welding radius) output will be weight, Area, Ix, Iy, Sx, Sy, Zx, Zy, ix, iy, cw, J, A_shear shear.

- Include welding between flange and web.

7.5 File manifest

| File | Language | Role |
| --- | --- | --- |
| optimized_beam_aisc.py | Python | AISC 360 grid-search optimizer |
| optimized_steel_beam.py | Python | EN 1993-1-1 grid-search optimizer (see 7.1 naming note) |
| bayesian_optimization.py | Python (numpy) | Phase 2 prototype — continuous-space BO refinement |
| CIPBeamOptimizer.jsx | React / JSX | Interactive calculator, Claude/React-environment artifact |
| CIP_Beam_Optimizer.html | HTML + React CDN | Standalone, build-free version for team distribution |

8. Assumptions & Limitations (Phase 1)

- Section property tables are indicative, drawn from common published values for a soldier-pile-relevant size range — verify against the current AISC Steel Construction Manual / EN 10365 tables before final design.

- LTB checks are simplified screening approximations (Section 6.3); they omit warping/torsion terms and should be replaced with full code equations for any case where the construction-stage check governs or for stamped final design.

- Strong-axis bending only; no biaxial bending, torsion, or seismic detailing provisions.

- Construction allowance tiers (50/75/100 mm) are practice-based defaults, not a codified requirement — confirm against the project specification (Section 4.2).

- No corrosion-loss (sacrificial section) reduction is applied; add a thickness deduction manually for long-term/permanent applications until this is added as a Phase 2 input.

- This is a screening/optimization tool to identify and rank candidate sections quickly — it does not replace a stamped structural calculation package.

9. Acceptance Criteria — Phase 1

- Both optimizer scripts run standalone with no external dependencies beyond the Python standard library (BO script requires numpy only) and complete a full library scan in under 1 second.

- Grid search and React calculator produce IDENTICAL admissible-section rankings for the same inputs (formulas mirrored 1:1 between Python and JavaScript — verified for the worked example in Section 4.3).

- React calculator renders and functions with zero build step (CIP_Beam_Optimizer.html opens directly in a browser).

- Every simplification (LTB, allowance tiers, section table provenance) is documented in-line in the code and in this PRD — no silent assumptions.

- Tool correctly flags "no admissible section" rather than silently returning an unsafe result when no catalog section fits or passes.

10. Roadmap

| Phase | Deliverable |
| --- | --- |
| Phase 1 (this PRD) | Grid-search optimizers (AISC + EN), React/HTML calculator, documented simplifications |
| Phase 2 | Full AISC F2 LTB (Cw, J, rts) and EN Annex Mcr; biaxial bending; production Bayesian layer for custom/built-up sections; corrosion-loss input; expanded section libraries |
| Phase 3 | Coupled cost optimization (drilling cost vs. steel weight vs. hole diameter); direct LPILE/GRLWEAP demand import (same pattern as the helical pile light-pole workflow); team deployment as an internal web tool |

11. References

- AISC 360-16, Specification for Structural Steel Buildings (Chapters F2, G2, H1).

- EN 1993-1-1:2005+A1, Eurocode 3: Design of steel structures — Part 1-1 (§6.2, §6.3).

- EN 10365, Hot rolled steel channels, I and H sections — dimensions and masses.

- Representative DOT drilled-in soldier pile special provisions (construction tolerance practice, referenced for Section 4.2 — project-specific specification governs).

- Internal: Helical Pile Optimization for Light Poles — PRD framework (lateral/moment-dominant design philosophy, dual-criteria termination logic).

- Internal: Directionally Optimized H-Beam (DOHB) — permanent retaining wall technical proposal.

- Include all check pass: calculation pipeline complete